NPTEL Online Certification Course

Industrial Engineering and Operations Research

Assignment Number 12: Detailed Solution

Indian Institute of Technology Guwahati

Q.1. In a single-period probabilistic inventory model (also known as the newsvendor model),

(A) demand remains fixed and predictable over the planning horizon

(B) demand during the lead time is a random variable with a known probability distribution

(C) inventory is replenished continuously and shortages are not permitted

(D) Safety stock is never required due to the one-time replenishment nature

Sol. In a single-period probabilistic inventory model, demand during the lead time is a random variable with a known probability distribution. This means that the exact demand during the selling season or period is uncertain or never known. Thus, a probability distribution related to demand is considered. Hence, option (B) is correct.

Q.2. In a deterministic inventory control model with instantaneous replenishment and no shortages,

(A) demand varies randomly over time and follows a probability distribution

(B) demand is time-independent and follows a known probability distribution

(C) demand rate is known, constant, and independent of inventory policy

(D) replenishment occurs at a finite rate and lead time is random

Sol. In a deterministic inventory control model with instantaneous replenishment and no shortages, the demand rate is known, constant, and independent of inventory policy. This is the assumption behind the basic Economic Order Quantity (EOQ) model. Hence, option (C) is correct.

Q.3. A basic inventory control model provides Economic Order Quantity (EOQ) as 6000 units, but purchase manager orders only 5000 units. The percentage increase in the inventory cost with respect to the minimum inventory cost is

(A) 0.7%   (B) 2.3%   (C) 4.5%   (D) 1.7%

Sol.

Given,

EOQ, Q* = 6000 units.

Order quantity, Q = 5000 units,

For a basic EOQ model, the total inventory cost TC is

|  | (1) |
| --- | --- |

where D is the annual demand, S is the setup cost per order and H is the holding cost per unit per year.

At EOQ (Q = Q*), the total cost is minimum, Eq. (1) becomes

At EOQ, ordering cost = holding cost, i.e.,

Thus,

Now, Eq. (1) can be written as

The percentage increase in cost is

Hence, option (D) is correct.

Q.4. In which of the following inventory models is safety stock most relevant and purposefully included to mitigate uncertainty?

(A) Newsvendor (single-period) model with perishable items

(B) Classical inventory model with instantaneous replenishment and no stock-outs

(C) Economic Order Quantity (EOQ) model with constant demand and zero lead time

(D) Probabilistic inventory model with uncertain demand or lead time

Sol. In probabilistic inventory models, demand or lead time is random. Thus, stock is purposefully included as a buffer to mitigate the risk of stock-out caused by unpredictable variations in demand or lead time. Hence, option (D) is correct.

Q.5. A company needs to plan the production of a product over a 6-month planning horizon. The monthly demand is shown in the table. Setup cost is Rs. 300 per order and the holding cost is Rs. 4 per unit per month.

| Month | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| Demand (units) | 100 | 110 | 80 | 60 | 90 | 70 |

As per Silver-Meal heuristics, the number of units produced in second month is

(A) 110       (B) 190       (C) 220      (D) 260

Sol.

Given,

Setup cost (per order), S = Rs. 300

Holding cost per unit per month, h = Rs. 2.

At t = 2 (month 2), demands: D2 = 110, D3 = 80, D4 = 60, D5 = 90, D6 = 70

We compute the average cost per month (AC) for producing in month t to cover demands up to month t+k, until the average cost per month increases for the first time.

Holding cost is calculated for the extra months we hold inventory for later demands.

Step 1: Start from month 1:

Produce for one month:

Produce for two months:

Hence, we produce only for month 1.

Step 2: Start from month 1:

Compute for n = 1

(no holding cost here)

Compute for n = 2

Compute for n = 3

The average cost decreases up to n = 2, but increases for n = 3.

Hence, optimal order covers n = 1 and n = 2 (order 110 + 80 = 190 units in second month)

Hence, option (B) is correct.

Q.6. A company faces the following monthly demands for a product over a 4-month horizon:

| Month | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Demand (units) | 120 | 140 | 80 | 70 |

Setup cost (per order) is ₹200 and holding cost is ₹2 per unit per month. Using the Wagner-Whitin algorithm, what is the minimum total cost (setup + holding) to meet all the demands without backlogging?

(A) ₹1000    (B) ₹1200    (C) ₹740     (D) ₹950

Sol.

Given,

Setup cost (per order), S = 200

Holding cost per unit per month, h = ₹2,

Demands: D1 = 120, D2 = 140, D3 = 80, D4 = 70.

Initialize by setting C(0) = 0.

Step 1: Calculate cost of all options

Cost of ordering in period 1 to cover demands up to period 1:

f(1, 1) = 200 + 0 = 200

Cost of ordering in period 1 to cover demands up to period 2:

f(1, 2) = 200 + 2 × 140 = 480

Cost of ordering in period 1 to cover demands up to period 3:

f(1, 3) = 200 + 2 × (1×140 + 2 × 80) = 800

Cost of ordering in period 1 to cover demands up to period 4:

f(1, 4) = 200 + 2 × (1×140 + 2 × 80 + 3 × 70) = 1220

Cost of ordering in period 2 to cover demands up to period 2:

f(2, 2) = 200 + 0 = 200

Cost of ordering in period 2 to cover demands up to period 3:

f(2, 3) = 200 + 2 × 80 = 360

Cost of ordering in period 2 to cover demands up to period 4:

f(2, 4) = 200 + 2 × (1 × 80 + 2 × 70) = 640

Cost of ordering in period 3 to cover demands up to period 3:

f(3, 3) = 200 + 0 = 200

Cost of ordering in period 3 to cover demands up to period 4:

f(3, 4) = 200 + 2 × 70 = 340

Cost of ordering in period 4 to cover demands up to period 4:

f(4, 4) = 200 + 0 = 200

Step 2: Dynamic programming

C(1) = min[C(0) + f (1, 1)] = min [200] = 200

C(2) = min[C(0) + f (1, 2), C(1) + f (2, 2)] = min [480, 200 + 200] = 400

C(3) = min[C(0) + f (1, 3), C(1) + f (2, 3), C(2) + f (3, 3)]

= min [800, 200 + 360, 400 + 360]

= min[800, 560, 760] = 560

C(4) = min[C(0) + f (1, 4), C(1) + f (2, 4), C(2) + f (3, 4), C(3) + f (4, 4)]

= min [1220, 200 + 640, 400 + 340, 560 + 200]

= min [1220, 840, 740, 760] = 740

Minimum total cost = ₹740

Hence, option (B) is correct.

Q.7. In which of the following inventory scenarios the newsvendor model is the most appropriately applied?

(A) Continuous review models with fixed order quantity and reorder point

(B) Multi-period inventory systems with predictable and constant demand

(C) Single-period inventory decisions under uncertain and variable demand

(D) Inventory systems where both demand and lead time are deterministic

Sol. The newsvendor model is designed for situations where a single order must be placed for a product (meaning decisions are made for a single selling period) with an uncertain, variable demand before the sales period begins. The unsold items cannot be carried over to the next period, and any leftover items are disposed of at the end of the period. Hence, option (C) is correct.

Q.8. The demand of a product varies uniformly between 0 to 20 units. The reorder point is 10 units. The expected shortage is

(A) 1 unit    (B) 1.5 units      (C) 2.5 units        (D) 4 units

Sol.

Given,

Reorder point, R = 10

Expected shortage is calculated as

Hence, option (C) is correct.

Q.9. The daily demand of a product is 140 units with standard deviation of 10 units.  The lead time is 2 days. The mean and standard deviation of the demand during the lead period is

(A) 280, 28 units, respectively

(B) 100, 40 units, respectively    
(C) 100, 20 units, respectively

(D) 280, 14 units, respectively

Sol.

Given,

Daily demand, D = 140 units,

Standard deviation,  = 10,

Lead time, L = 2.

Mean demand during lead period = DL= 1402= 280 units

Standard deviation during lead period = units

Hence, option (D) is correct.

Q.10. In a probabilistic model, the expected demand is 400, holding cost is Rs. 80 per unit and standard deviation is 20 units. The safety stock required for a service level of 90% (z = 1.28) is

(A) 40.8		(B) 52.5	(C) 60.5		(D) 25.6

Sol.

Given,

Standard deviation, σ = 20 units,

z = 1.28.

Safety stock = zσ  =1.2820 = 25.6 units

Hence, option (D) is correct.

Q.11. A company faces normally distributed demand during lead time with a mean (μ) of 400 and standard deviation (σ) of 45. To achieve a service level of 95% (z = 1.645), the reorder point (ROP) is

(A) 400	(B) 474	(C) 540	(D) 626

Sol.

Given,

Mean, µ = 400,

Standard deviation, σ = 45,

z = 1.645.

Reorder point (ROP) = µ+zσ  =400+1.64545 = 474.025 units

Hence, option (B) is correct.

Q.12. A shopkeeper sells walnuts priced at ₹1500 per kg. The daily demand for walnuts follows a uniform continuous distribution from 0 to 10 kg. However, the shopkeeper stocks only 9 kg of walnuts per day. Assuming that sales cannot exceed available stock, the expected sales revenue (in ₹) for the day is

(A) ₹6000       (B) ₹3950     (C) ₹7425      (D) ₹6150

Sol.

Given,

Selling price = ₹1500 per kg,

Stock level, Q = 9,

Daily demand, D ∼ U(0, 10),

For uniform distribution on [0, 10], = 1/10, 0 ≤ t ≤ 10.

For a continuous random variable D,

For Q = 9,

Expected revenue, E[revenue] = 1500 × E[min(D, Q)] = 1500 × 4.95 = ₹7425.

Hence, option (B) is correct.

Q.13. A shopkeeper sells drill bits, each priced at ₹1250. The daily demand for drill bits follows a uniform discrete distribution over the integers from 1 to 10. However, the shopkeeper stocks only 9 drill bits per day. Assuming that sales cannot exceed the available stock, the expected sales revenue (in ₹) for a day is

(A) ₹4290       (B) ₹5950     (C) ₹6750     (D) ₹4500

Sol.

Given,

Selling price per drill bit = ₹1250

Stock level, Q = 9

Daily demand, D = discrete uniform {1, 2, 3, …, 10}

So, P(D = k) = 1/10 for k =1, 2,…, 10.

Expected sales quantity is

Expected revenue, E[revenue] = 1250 × E[min(D, Q)] = 1250 × 5.4 = ₹6750.

Hence, option (A) is correct.

Q.14. Let

The first derivative of F(x) with respect to x is given by

(A) cos(x4) sin(x2)      (B) 2xsin(x4) cos(x4)        (C) 2xcos(x4)cos(x2)        (D) xcos(x4)

Sol. Use Leibniz's integral rule

here , , and .

Hence, option (C) is correct.

Q.15. Let

The first derivative of F(x) with respect to x is given by

(A)            (B)           (C)        (D)

Sol. Use Leibniz's integral rule

here , , and .

Hence, option (B) is correct.

Q.16. A multi-period inventory model is used to

(A)  minimize the setup cost of inventory over a single period

(B)  minimize the total cost of inventory over a single production

(C)  determine the optimal order quantity for a single order

(D)  minimize the total cost of inventory over multiple periods

Sol.  A multi-period inventory model considers several time periods and aims to minimize the total cost—including setup (or ordering), holding, and shortage costs over the entire planning horizon. Hence, option (D) is correct.

Q.17. The optimum order quantity in a single-period model without the setup cost is 100 units. When the setup cost is included, the optimal order quantity will be

(A) always less than 100

(B) always more than 100

(C) 100 only

(D) dependent on setup cost

Sol. In a single-period model, the optimal order quantity is independent of the setup cost. Including a setup cost will not change the optimal order quantity of 100 units. Hence, option (C) is correct.

Q.18. The average daily demand of a product is 50 units. The lead time is 5 days and the safety stock maintained is 30 units. The reorder point is

(A) 250 units		(B) 1500 units		(C) 150 units		(D) 280 units

Sol. Demand during lead time = 505=250

ROP = 250+30=280

Hence, option (D) is correct.

Q.19. A company experiences daily demand for a product that follows a normal distribution with a mean of 200 units and a standard deviation of 30 units. The lead time is 6 days. The company wants to maintain a 95% cycle service level (z = 1.645). The safety stock (in units) is

(A) 151        (B) 211          (C) 121         (D) 331

Sol.

Given,

Mean,  = 200 units,

Standard deviation,  = 30 units,

Lead time = 6 days,

z = 1.645.

Now, standard deviation during lead time, =

Safety stock =  units

Hence, option (C) is correct.

Q.20. A seasonal product has demand following a normal distribution with mean of 400 units and standard deviation of 80 units.  Selling price is ₹800, purchase price is ₹300, and salvage value is ₹200. The optimal order quantity (nearest integer) is

(A) 459         (B) 572         (C) 512     (D) 551

Sol.

Given,

Mean, = 400 units,

Standard deviation,  = 80 units,

Selling price per unit, p = ₹800,

Purchase price per unit, c = ₹300,

Salvage value, s = ₹150.

Now,

Cost of shortage, = p–c = 800–300 = ₹500

= = 300 – 150 = ₹150

Now, the z value has to be evaluated that yields a probability of 0.769. Using MS Excel command NORMSINV(0.769), the z value is 0.7356.

Optimal order quantity =

Thus, optimal order quantity is 459 (nearest integer).

Hence, option (A) is correct.