NPTEL Online Certification Course

Industrial Engineering and Operations Research

Assignment Number 5: Detailed Solution

Indian Institute of Technology Guwahati

Q.1. Which of the following is NOT a goal of line balancing?

(A) Equalize workloads	(B) Reduce cost
(C) Increase idle time		(D) Maximize efficiency

Sol.  Line balancing aims to reduce, not increase, idle time. Hence, option (C) is correct.

Q.2 Which of the following is/are correct statement(s) of line balancing in a production system?

(A) It improves productivity and optimizes resource utilization.
(B) It reduces cycle time and helps eliminate bottlenecks.
(C) It maximizes idle time to avoid worker overload.
(D) It helps determine manpower requirement and production speed.

Sol.  Line balancing increases efficiency and ensures better use of manpower and resources. Reduces cycle time and helps eliminate bottlenecks. It minimizes delays and improves process flow and maximizes idle time to avoid worker overload. Line balancing aims to minimize idle time, not maximize it. It also helps to determine manpower requirements and production speed 
It assists in workforce planning and production rate determination. Hence, options (A), (B) and (D) are correct.

Q.3. In basic Assembly Line Balancing models, which of the following assumptions are commonly made?

(A) Task processing times are deterministic and known in advance, and task precedence relationships are predefined.

(B) A task may be split among multiple workstations, and parallel workstations are permitted.

(C) Worker learning effects, fatigue, and variability in task performance are neglected, and task times are assumed fixed.

(D) No intermediate Work-in-Progress (WIP) buffers are considered between consecutive workstations.

Sol. Basic Assembly Line Balancing models generally assume deterministic task times, fixed precedence relationships, and neglect worker-related variability such as learning and fatigue. They also typically assume a paced line without intermediate WIP buffers, whereas task splitting and parallel stations are usually not allowed in the simple model. Hence, options (A), (C) and (D) are correct.

Q4. A line contains two stations with cycle times of 12 minutes and 18 minute, respectively. The smoothness index is

(A) 10 minutes   (B) 6 minutes     (C) 4 minutes    (D) 9.7 minutes

Sol.

Given,

Station 1:  = 12 minutes

Station 2:  =18 minute

The smoothness index is given by

minutes

Hence, option (B) is correct.

Q.5. Which of the following statement(s) regarding the importance of contribution is/are correct?

(A) A product with positive contribution may still not generate overall profit if fixed costs are not fully covered.
(B) If an organization is already profitable, additional sales generally increase profit by the amount of contribution per unit.
(C) When an organization incurs losses, additional sales with positive contribution can reduce losses.
(D) Positive contribution necessarily implies that net profit is positive for the organization.

Sol.  A positive contribution covers variable costs and contributes toward fixed costs, but profit depends on whether fixed costs are fully recovered. Once fixed costs are covered, additional contributions directly increase profit. Contribution helps absorb fixed costs, thereby lowering losses. A firm may still incur losses if fixed costs exceed total contribution. Hence, options (A), (B) and (C) are correct.

Q.6. A company makes 1200 units of a product with a selling price of ₹105 per unit and variable costs of ₹88 per unit. The fixed cost for the period is ₹28000. The total contribution is

(A) ₹98,000     (B) ₹48,400     (C) ₹20,400     (D) ₹8,400

Sol.

Given,

Units produced = 1200

Selling price = ₹105

Variable cost = ₹88

Total sales = 1200×105 = ₹1,26,000

Total variable cost = 1200×88 = ₹1,05,600

Total contribution = Sales – Variable cost = 126000 – 105600 = ₹20,400

Hence, option C is correct.

Q.7. A company is making 1200 units of a product at a variable cost of ₹92 per unit and selling at a price of ₹180 per unit. The fixed cost is ₹90,000. Which one of the following statements is TRUE?

(A) The company is selling the product at profit.

(B) The company is selling the product at loss. Making and selling 1000 units will further enhance the loss.

(C) The company is selling the product at loss. Making and selling 1000 units will reduce the loss to zero.

(D) The company is selling the product at profit. Making and selling only 1,000 units will also yield profit.

Sol.  Given,

Selling price = ₹180

Fixed cost = ₹50000

Variable cost = ₹92

For 1200 units:

Total sales = 1200×180 = ₹2,16,000

Total variable cost = 1200×92 = ₹1,10,400

Total contribution = Sales – Variable cost = 2,16,000 – 1,10,400 = ₹1,05,600

Profit = Contribution – Fixed cost = 1,06,000 – 90,000 = ₹16,000 (profit)

For 1000 units:

Total sales = 1000×180 = ₹1,80,000

Total variable cost = 1000×92 = ₹92,000

Total contribution = Sales – Variable cost = 1,80,000 – 92,000 = ₹88,000

Profit = Contribution – Fixed cost = 88,000 – 90,000 = – ₹2,000 (loss).

Hence, option A is correct.

Q.8. A company is making 800 units of a product at a variable cost of ₹250 per unit and selling at a price of ₹1,000 per unit. The fixed cost is ₹42,000. The Profit-Volume (PV) ratio is

(A) 28%   (B) 75%    (C) 31%    (D) 25%

Sol.

Given,

Units produced = 800

Variable cost = ₹250

Selling price = ₹1,000

Fixed cost = ₹42,000

Contribution per unit = Selling price per unit – variable cost per unit = 1,000 – 250 = ₹750

Profit-Volume (PV) ratio is given by

Hence, option (B) is correct.

Q.9. A company is making 1500 units of a product at a variable cost of ₹700 per unit and selling at a price of ₹1,250 per unit. The fixed cost is ₹80,000. The breakeven point in value is

(A) ₹145,455        (B) ₹142,857         (C)  ₹80,000     (D) ₹51,282

Sol.

Given,

Units produced = 1500

Selling price = ₹1,250

Variable cost = ₹700

Fixed cost = ₹80,000

Contribution per unit = Selling price per unit – variable cost per unit = 1,250 – 700 = ₹550

Break-even point in value is given by

= ₹145,455

Hence, option (A) is correct.

Q.10. For a product, the fixed cost is ₹40,000, the variable cost is ₹15,000, and the PV ratio is 35%. The breakeven point is

(A) ₹71,728   (B) ₹42,857   (C) ₹114,286   (D) ₹57,142

Sol.

Given,

Variable cost = ₹20,000

Fixed cost = ₹40,000

PV ratio = 0.35

Break-even point is given by

= ₹114,286

Hence, option (C) is correct.

Q.11. A manufacturing plant operates a high-cost critical machine. Which of the following actions represent appropriate production management strategies regarding machine capacity utilization?

(A) If the machine is significantly under-utilized, management may seek additional production orders or diversify product demand to improve capacity utilization.

(B) If the machine consistently operates beyond its effective capacity, management may consider outsourcing part of the workload or installing additional machines.

(C) Machine utilization should always be maintained at 100%, irrespective of maintenance requirements or production variability.

(D) Reducing preventive maintenance activities is an effective method for increasing long-term machine availability and productivity.

Sol.  Effective Production Management aims at balancing machine utilization with reliability and operational flexibility. Excessive utilization or neglect of maintenance may reduce productivity and increase breakdown risk. Hence, options (A) and (B) are correct.

Q.12. Which one of the following is a quantifiable parameter in the decision-making?

(A) Customer goodwill			(B) Market conditions
(C) Company’s strategic focus		(D) Contribution margin ratio

Sol. Contribution margin ratio is a measurable financial metric, unlike the others, which are qualitative factors. Hence, option (D) is correct.

Q.13. The period of the Second Industrial Revolution is

(A) 19151918 CE       (B) 17601840 CE   (C) 18251900 CE    (D) 18701914 CE

Sol.  The Second Industrial Revolution is generally considered to have taken place between 1870 and 1914, characterised by rapid industrialisation, mass production, electricity, steel, petroleum, and advances in communication and transportation. Hence, option (D) is correct.

Q.14. The Internet was invented in

(A) 1950s         (B) 1970s    (C) 1990s      (D) 2010s

Sol. The Internet is generally considered to have come into existence around 1973, when the development of the TCP/IP protocol began, enabling different computer networks to interconnect and form the basis of the modern Internet. Hence, option (B) is correct.

Q.15. In Cloud Computing, the three primary cloud service delivery models are

(A) Infrastructure as a Service (IaaS), Platform as a Service (PaaS), and Software as a Service (SaaS)

(B) Network as a Service (NaaS), Hardware as a Service (HaaS), and Security as a Service (SECaaS)

(C) Data Storage as a Service (DSaaS), Computing as a Service (CaaS), and Internet as a Service (INaaS)

(D) Program as a Service (PRaaS), Device as a Service (DaaS), and Server as a Service (SVaaS)

Sol.  IaaS, PaaS, and SaaS constitute the standard and widely accepted cloud service models. They respectively provide computing infrastructure, application development platforms, and end-user software services over the Internet. Option A is correct.

Q.16. With reference to numerical methods used for estimating Residual Stress, which of the following statements are correct?

(A) Accurate prediction of residual stresses generally requires elastic–plastic material analysis.

(B) Updated Lagrangian formulations are computationally intensive for large deformation problems.

(C) Pure Eulerian formulations are generally less suitable for residual stress prediction in solid deformation processes.

(D) In Eulerian formulations, accumulation of discretization errors may affect solution accuracy.

Sol.  Residual stresses arise after unloading, so elastic-plastic analysis is necessary to capture permanent deformation and elastic recovery. Updated Lagrangian methods involve continuous remeshing/updating of geometry during deformation, which increases computational cost. The Eulerian method is generally less effective for residual stress estimation because tracking material history and unloading effects is difficult. Accumulation of discretization errors may affect solution accuracy in all methods.

Hence, options (A), (B), (C) and (D) are correct.

Q.17. In Radial Basis Function methods, which of the following radial basis functions exhibits exponential decay with distance?

(A) Multiquadric (B) Gaussian   (C) Thin-Plate Splines  (D) Inverse multiquadric

Sol. The Gaussian Radial Basis Function (RBF) is characterized by an exponential decay form, generally written as:

where r is the radial distance from the center point and ck  is the shape parameter. Since the function contains an exponential term with a negative squared distance, the value of ‘g’ decreases rapidly as ‘r’ increases. This is known as exponential decay, which is the characteristic property of the Gaussian RBF. Hence, option (B) is correct.

Q.18. With reference to Molecular Dynamics simulation, which of the following statement(s) is/are correct?

(A) It provides fundamental insight into atomic-scale deformation mechanisms.

(B) It can simulate material behavior without assuming the evolution path of deformation mechanisms in advance.

(C) It is computationally expensive for simulations involving large spatial and temporal scales.

(D) It is particularly useful when the underlying deformation mechanisms are not fully understood.

Sol. Molecular Dynamics simulations track atomic interactions directly and are therefore valuable for studying complex or poorly understood deformation phenomena. However, they are computationally intensive and are generally limited to relatively small length and time scales. All options are correct.

Q.19. Which one of the following features contributes to the sustainability of powder-based manufacturing?

(A) Fine resolution                                          (B) Recycling of powders

(C) Slow process speed                                    (D) Temperature sensitivity

Sol.  Fine resolution improves precision and product quality, but it does not directly contribute to sustainability. During powder-based manufacturing, not all powder is fused or used in the final product. The remaining powder can often be recovered and reused. A slower process is generally considered a limitation, as it may increase production time and energy consumption rather than improve sustainability. Sensitivity to temperature fluctuations is a process limitation, not a sustainability feature. Hence, option (B) is correct.

Q.20. Which of the following contribute(s) to resilience in Industry 5.0?

(A) Adaptation to pandemics and supply chain disruptions
(B) Smart automation
(C) Human flexibility in decision-making
(D) Dependence on a single rigid production system

Sol.  Industry 5.0 systems are designed to handle disruptions. Smart automation improves adaptability. Human flexibility supports resilience. Resilience requires flexibility, not rigidity. Hence, options (A), (B) and (C) are correct.