NPTEL Online Certification Course

Industrial Engineering and Operations Research

Assignment Number 7: Detailed Solution

Indian Institute of Technology Guwahati

Q.1. Operations Research unit at the Indian Statistical Institute, Kolkata was established in the year

(A) 1949            (B) 1953       (C) 1957         (D) 1960

Sol. Operations Research (OR) unit at the Indian Statistical Institute, Kolkata was established in the year 1953. The purpose was to use OR in national planning and survey.  Hence, option (B) is correct.

Q.2. Minimize  is equivalent to

(A) Maximize                (B) Minimize

(C) Maximize               (D) Minimize

Sol. For any objective function f (x, y), max f ⟺ min (–f). Minimizing is equivalent to maximizing the expression , which is also equivalent to minimizing . Hence, option (B) is correct.

Q.3. In a linear programming problem with 9 decision variables and 8 equality constraints, the number of basic solutions is

(A) 10     (B) 45    (C) 9      (D) 12

Sol. In the algebraic solution space (defined by  equations, , where n = number of decision variables and m = number of equality constraints), basic solutions correspond to the corner points in the graphical solution space. The basic solutions are determined by setting n – m (= 9 – 8 = 1) variable(s) equal to zero and solving the m equations for the remaining m (= 8) variables. The number of possible basic solutions (corner points) is

Hence, option (C) is correct.

Q.4. Which one of the following best defines a convex set?

(A) A set in which the entire line segment joining any two points of the set lies completely within the set.
(B) A set in which the line joining any two points of the set intersects the set at more than one point.
(C) A set in which the midpoint of any two points in the set always lies within the set.
(D) A set in which every boundary point can be expressed as a convex combination of interior points.

Sol. A set S is convex if every point on the line segment connecting x and y is in S as shown in figure.

| A convex set | A non-convex set |
| --- | --- |

Hence, option (A) is correct.

Q.5. In the problem,

Maximize 3X1 + 2X2

subject to

X1 + 3X2  30                        (1)

3X1 + X2  25                         (2)

X1  0, X2  0,

the number of basic feasible solutions is

(A) 0    (B) 3   (C) 4    (D) 6

Sol. The problem is converted to standard form by adding slack variables S1, S2. The solution space of the linear programming is now represented by m = 2 equations and n = 4 variables (X1, X2, S1, S2):

X1 + 3X2 + S1 = 30       (1)

3X1 + X2 + S2 = 25       (2)

X1, X2, S1, S2  0

The basic solutions are determined by setting n – m (= 4 – 2 = 2) variables equal to zero and solving for the remaining m (= 2) variables. The total number of possible basic solution is

.

- Set X1 = 0 and X2 = 0 (X1, X2: non-basic variables),

Eq. (1): S1 = 30,

Eq. (2): S2 = 25. (S1, S2: basic variables)
Basic solution: (S1, S2) = (30, 25) feasible

- Set X1 = 0 and S1 = 0 (X1, S1: non-basic variables),

Eq. (1): X2 = 10,

Eq. (2): X2 + S2 = 25 ⇒ S2 = 25 − 10 = 15. (X2, S2: basic variables)

Basic solution: (X2, S2) = (10, 15) feasible

- Set X1 = 0 and S2 = 0 (X1, S2: non-basic variables),

Eq. (2): X2 = 25,
Eq. (1): S1 = 30 − 3X2 = 40 – 75 = −35. (X2, S1: basic variables)
Basic solution: (X2, S1) = (25, −35) infeasible (since S1<0)

- Set X2 = 0 and S1 = 0 (X2, S1: non-basic variables)

Eq. (1): X1 = 30,
Eq. (2): S2 = 25 − 3X1 = 25 – 90 = −65. (X1, S2: basic variables) 
Basic solution: (X1, S2) = (30, −65) infeasible (since S2<0)

- Set X2 = 0 and S2 = 0 (X2, S2: non-basic variables),

Eq. (2): 3X1 = 25 ⇒ X1 = 25/3,
Eq. (1): S1 = 30 − X1 = 30 – 25/3 = 65/3. (X1, S1: basic variables) 
Basic solution: (X1, S1) = (25/3, 65/3) feasible

- Set S1 = 0 and S2 = 0 (S1, S2: non-basic variables),

Eq. (1): X1 + 3X2 = 30,
Eq. (2): 3X1 + X2 = 25. (X1, X2: basic variables)

Solve to get basic solution: (X1, X2) = (40, 24) feasible

Out of 6 possible basic solutions, 4 are feasible. Hence, option (C) is correct.

Q.6. A company produces two products, P1 and P2. Each unit of P1 requires 1 hour on Machine A and 1 hour on Machine B. Each unit of P2 requires 2 hours on Machine A and 3 hours on Machine B. Machine A has 100 hours, and Machine B has 90 hours available. The profit is ₹30 per unit of P1 and ₹40 per unit of P2. The maximum profit is

(A) ₹2000        (B) ₹2200        (C) ₹2400          (D) ₹2700

Sol. The optimum solution is the feasible basic solution (corner point) with the best objective value. The problem is expressed as:

Maximize Z = 30X1 + 40X2

Subject to

(Machine A)

(Machine B)

Convert to standard form by adding slack variables S1, S2. The solution space of the LP is now represented by the following m = 2 equations and n = 4 variables (X1, X2, S1, S2):

X1 + 2X2 + S1 = 100       (1)

X1 + 3X2 + S2 = 90         (2)

X1, X2, S1, S2  0

The basic solutions are determined by setting n – m (= 4 – 2 = 2) variables equal to zero and solving for the remaining m (= 2) variables.

- Set X1 = 0 and X2 = 0 (X1, X2: non-basic variables),

Eq. (1): S1 = 100,

Eq. (2): S2 = 90. (S1, S2: basic variables)
Basic solution: (S1, S2) = (100, 90) feasible

Z = 30X1 + 40X2 = 0

- Set X1 = 0 and S1 = 0 (X1, S1: non-basic variables),

Eq. (1): X2 = 50,

Eq. (2): 3X2 + S2 = 90 ⇒ S2 = 90 − 150 = −60. (X2, S2: basic variables)

Basic solution: (X2, S2) = (50, −60) infeasible (since S2<0)

- Set X1 = 0 and S2 = 0 (X1, S2: non-basic variables),

Eq. (2): X2 = 30,
Eq. (1): S1 = 100 − X2 = 100 – 60 = 40. (X2, S1: basic variables)
Basic solution: (X2, S1) = (30, 40) feasible

Z = 30X1 + 40X2 = 40×30=1200

- Set X2 = 0 and S1 = 0 (X2, S1: non-basic variables)

Eq. (1): X1 = 100,
Eq. (2): S2 = 90 − X1 = 90 – 100 = –10. (X1, S2: basic variables)
Basic solution: (X1, S2) = (100, –10) infeasible (since S2<0)

- Set X2 = 0 and S2 = 0 (X2, S2: non-basic variables),

Eq. (2): X1 = 90,
Eq. (1): S1 = 100 − X1 = 100 – 90 = 10. (X1, S1: basic variables) 
Basic solution: (X1, S1) = (90, 10) feasible (since S1<0)

Z = 30X1 + 40X2 = 30×90=2700

- Set S1 = 0 and S2 = 0 (S1, S2: non-basic variables),

Eq. (1): X1 + 2X2 = 100,
Eq. (2): X1 + 3X2 = 90. (X1, X2: basic variables)

Solve to get basic solution: (X1, X2) = (120, –10) infeasible (since X1<0)

The maximum profit is ₹2700, achieved by producing 90 units of P1 and 0 units of P2. Hence, option (D) is correct.

Q.7. Consider the following linear programming problem:

Maximize z = 5x + 8y

subject to

x + 2y  10,

x + y  3,

x  0, y  0.

This problem has

(A) redundant constraint     (B) feasible solution    (C) no solution    (D) unique solution

Sol.

Constraint 1: x + 2y  10

If x=0, then y=5

If y=0, then x=10

This represents the region above and to the right of the line connecting (10,0) and (0,5).

Constraint 2: x + y  3

If x=0, then y=3

If y=0, then x=3

This represents the region below and to the left of the line connecting (3,0) and (0,3). Also, the non-negativity constraints restrict the solution to first quadrant.

As shown in graph, there are no common points that satisfy all constraints simultaneously. There exists no solution. Hence, option (C) is correct.

Q.8. Consider the following linear programming problem:

Maximize z = 3x + 7y

subject to

2x + y  8,

x + y  15,

x  0, y  0.

This problem has

(A) infeasible solution      (B) feasible solution    (C) redundant constraint  (D) no solution

Sol.

Constraint 1: 2x + y  8

If x=0, then y=8

If y=0, then x=4

This represents the region above and right of the line connecting (4,0) and (0,8).

Constraint 2: x + y  15

If x=0, then y=15

If y=0, then x=15

This represents the region below and left of the line connecting (15,0) and (0,15).

As shown in graph, the overlapping region bounded by these conditions forms a non-empty, enclosed polygon. Since a common region exists, the problem has a feasible solution. Hence, option (B) is correct.

Q.9. Consider the following optimization problem:

Maximize

subject to

The standard linear programming problem will contain

(A) 3 slack variables

(B) 2 slack and 1 surplus variables

(C) 2 surplus and 1 slack variables

(D) 3 surplus variables

Sol. Conversion from () to (=) is achieved by adding a nonnegative slack variable from the left-hand side of the inequality. Conversion from () to (=) is achieved by subtracting a nonnegative surplus variable from the left-hand side of the inequality.

Constraint 1: → add slack variable →,

Constraint 2: → subtract surplus variable →,

Constraint 3:→ subtract surplus variable → ,

The problem contains 1 slack and 2 surplus variables. Hence, option (C) is correct.

Q.10. Consider the following optimization problem:

Maximize z = 4A + 5B + 6C + 7D + 8E

Subject to

Constraint 1:    A + B + C + D + E 10

Constraint 2:    D + E  5

Constraint 3:    C  A+B

Constraint 4:   A + 0.2B + 0.4C  3

A, B, C, D, E  0.

If you convert this problem into a linear programming problem and solve it by simplex method, then optimal solution is obtained as A = 2, B = 0, C = 3, D = 0, E = 5. The active constraints are (A) constraints 1 and 4   (B) constraints 1 and 2   (C) constraints 2 and 4  (D) constraints 3 and 4

Sol. Substitute in all the constraints: A = 2, B = 0, C = 3, D = 0, E = 5.

Constraint 1:

A + B + C + D + E = 2 + 0 + 3 + 0 + 5 =10

Since the left-hand side equals the right-hand side, this constraint is active.

Constraint 2:

D + E = 0 + 5 = 5

Since the left-hand side equals the right-hand side, this constraint is active.

Constraint 3:

C – (A + B) = 3 – (2 + 0) = 1

Since the left-hand side is not equal to the right-hand side, this constraint is inactive.

Constraint 4:

A + 0.2B + 0.4C =2+0.20+0.43=3.2

Since the left-hand side is not equal to the right-hand side, this constraint is inactive.

Constraints 1 and 2 are active. Hence, option (B) is correct.

Q.11. Consider the problem of maximization of the following objective function:

Suppose for solving the problem, two artificial variables  and  are needed. A proper objective of the problem is

(A) Maximize

(B) Minimize

(C) Minimize

(D) Maximize

Sol. When artificial variables are introduced (in equations involving ≥ constraints), they must be forced out of the basis.

In the Big-M method:

  - For maximization problems, the artificial variables  and  are penalised in the objective function with , i.e., a  large negative penalty is introduced, where  is a sufficiently large positive value.

  - For minimization problems, the artificial variables  and  are penalised in the objective function with , i.e., a  large positive penalty is introduced, where  is a sufficiently large positive value.

Here,  is a maximization problem. Therefore, the proper objective of the problem is:

Maximize , where .

Hence, option (D) is correct.

Q.12. Maximize Z = 2x + 3y

Subject to

2x + 3y ≤ 9

0 ≤ x ≤ 3, 0 ≤ y ≤5

The problem

(A) has no feasible solution

(B) is unbounded

(C) has a unique optimal solution

(D) has multiple optimal solutions

Sol. Alternative optima can arise in the LP model when the objective function is parallel to a non-redundant binding constraint (i.e., a constraint that is satisfied as an equation at the optimal solution). Here, the objective function Z = 2x + 3y is parallel to the constraint 2x + 3y ≤ 9. Any point on the line segment 2x + 3y = 9 that lies along the edge of the feasible region represents an alternative optimum with the same objective value Zmax = 9. Hence, option (D) is correct.

Q.13. In simplex method, if the objective function increases indefinitely without violating constraints, the problem has

(A) infeasible solution    (B) unbounded solution    (C) multiple solution  (D) degeneracy

Sol. In simplex method, the objective function value increase (or decrease) infinitely without touching any boundaries when the feasible region is open (not enclosed). Such problems have unbounded solution. Hence, option (B) is correct.

Q.14. In linear programming problem, an unrestricted decision variable is represented as

(A) as the difference of two real numbers

(B) as the difference of two positive numbers

(C) as the difference of two non-negative numbers

(D) an unsigned variable

Sol. In linear programming, if a variable x is unrestricted in sign, it is spitted into two variables, i.e., , where

An unrestricted variable is expressed as the difference of two non-negative variables. Hence, option (C) is correct.

Q.15. While solving a maximization problem using the simplex method, the basic variables and their corresponding solution values at a particular iteration are x1 = 0, x2 = 2, S1 = 0, S2 = 3. Which one of the following is the most appropriate statement?

(A) Degeneracy occurs due to S1 = 0.

(B) The solution is degenerate because more than one variable is zero.

(C) The solution is degenerate because at least one basic variable is zero.

(D) This is not a degenerate solution.

Sol. In the Simplex iteration, a basic feasible solution is degenerate if at least one basic variable is equal to 0. Here, basic variables include x1 = 0 and S1 = 0. Therefore, the solution is degenerate. Hence, option (C) is correct.

Q.16. Consider the following linear programming problem:

Minimize

subject to

This problem can be solved

(A) either by two-phase method or by big M method

(B) by big M method but not by two phase method

(C) by two-phase method but not by big M method

(D) neither by two-phase method nor by big M method

Sol. Both equality (=) and inequality (≥) constraints require artificial variables, so the problem can be solved either by the two-phase method (phase I to remove artificial variables) or by the big-M method (penalize artificial variables in the objective). Hence, option (A) is correct.

Q.17. The following problem is solved using big M method:

Maximize Z = 3x + 5y − M R1 − M R2

subject to

x + y ​= 7

x – y = 3

x, y, R1, R2  0

The value of Z is

(A) 0    (B) 36   (C) 12    (D) 25

Sol.  Maximize Z = 3x + 5y − M R1 − M R2

subject to

x + y ​ + R1 = 7

x – y + R2 = 3

x, y, R1, R2  0

Introduce a large penalty M = 100 for the artificial variables R1 and R2.

Initial Big M Simplex Tableau:

| Basic | x | y | R1 | R2 | Solution |
| --- | --- | --- | --- | --- | --- |
| Z | −3 | −5 | 100 | 100 | 0 |
| R1 | 1 | 1 | 1 | 0 | 7 |
| R2 | 1 | −1 | 0 | 1 | 3 |

Make the Z-row consistent (Z-row = Z-row –100×(R1 + R2) row. The following simplex table is obtained:

| Basic | x | y | R1 | R2 | Solution |
| --- | --- | --- | --- | --- | --- |
| Z | −203 | −5 | 0 | 0 | −1000 |
| R1 | 1 | 1 | 1 | 0 | 7 |
| R2 | 1 | −1 | 0 | 1 | 3 |

The most negative coefficient in the Z-row is –203 (corresponding to x​), so x​ enters the basis.

R1: (7/1) = 7, R2 = (3/1) = 3. By minimum ratio rule, R2 is leaving.

Considering x as entering variable and R2 as leaving variable, the following simplex table is obtained.

| Basic | x | y | R1 | R2 | Solution |
| --- | --- | --- | --- | --- | --- |
| Z | 0 | −208 | 0 | 205 | −391 |
| R1 | 0 | 2 | 1 | −1 | 4 |
| x | 1 | −1 | 0 | 1 | 3 |

Perform row operations to make the coefficients of basic variable x equal to zero in Z and R1 row. Considering y (with the most negative Z-row coefficient = −208) as entering variable and R1 as leaving variable (using minimum ratio rule), the following simplex table is obtained.

| Basic | x | y | R1 | R2 | Solution |
| --- | --- | --- | --- | --- | --- |
| Z | 0 | 0 | 104 | 99 | 25 |
| y | 0 | 1 | 1/2 | −1/2 | 2 |
| x | 1 | 0 | 1/2 | 1/2 | 5 |

Perform row operations to make the coefficients of basic variable y equal to zero in Z and x row.

Since there are no negative values in the Z row, we have reached the optimal solution.

The optimal solution is x = 5 and y = 2. Maximum Z = 3x + 5y = 3×5 + 5×2 = 25.

However, there is a short-cut. We know that ultimately artificially variables have to be zero. Hence, Z = 3x + 5y. Solving two equality constraints x + y ​= 7 and x – y = 3, the value of Z is obtained as 25. Hence, option (D) is correct.

Q.18. The initial simplex table in the first phase of two-phase simplex method is

| Basic | x₁ | x₂ | R | RHS |
| --- | --- | --- | --- | --- |
| Objective row | 0 | 0 | 1 | 0 |
| R | 1 | 1 | 1 | 5 |

The objective of the first phase is the minimization of R. The consistent table is

(A)

| Basic | x₁ | x₂ | R | RHS |
| --- | --- | --- | --- | --- |
| Objective row | 1 | 1 | 0 | 5 |
| R | 1 | 1 | 1 | 5 |

(B)

| Basic | x₁ | x₂ | R | RHS |
| --- | --- | --- | --- | --- |
| Objective row | 1 | 1 | 0 | 5 |
| R | 1 | 1 | 1 | 5 |

(C)

| Basic | x₁ | x₂ | R | RHS |
| --- | --- | --- | --- | --- |
| Objective row | 1 | 1 | 1 | 5 |
| R | 1 | 1 | 1 | 5 |

(D)

| Basic | x₁ | x₂ | R | RHS |
| --- | --- | --- | --- | --- |
| Objective row | 1 | 1 | 0 | 0 |
| R | 1 | 1 | 1 | 5 |

Sol. The table can be made consistent by adding the objective row and the R row in the objective row (eliminating −1 under the R column in the objective row).

| Basic | x₁ | x₂ | R | RHS |
| --- | --- | --- | --- | --- |
| Objective row | 1 | 1 | 0 | 5 |
| R | 1 | 1 | 1 | 5 |

Hence, option (B) is correct.

Q.19. At the end of first phase of the simplex, the following table is obtained with r as the objective function:

| Basic | A | B | C | D | E | R1 | R2 | Solution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r | 0 | 0 | 0 | 0 | 0 | −1 | −1 | 3 |
| B | 0 | 1 | −1 | 0 | 1 | 1 | −1 | 4 |
| D | 0 | 0 | −5 | 1 | 7 | 5 | −7 | 15 |
| R1 | 1 | 0 | 1 | 0 | −2 | −1 | 2 | 3 |

This problem has

(A) feasible solution     (B) infeasible solution  (C) redundant constraint  (D) no solution

Sol.

| Basic | A | B | C | D | E | R1 | R2 | Solution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| r | 0 | 0 | 0 | 0 | 0 | −1 | −1 | 2 |
| B | 0 | 1 | −1 | 0 | 1 | 1 | −1 | 4 |
| D | 0 | 0 | −5 | 1 | 7 | 5 | −7 | 14 |
| R1 | 1 | 0 | 1 | 0 | −2 | −1 | 2 | 3 |

Table shows that the artificial variable R1 is 3 (positive), meaning that the problem is infeasible. Hence, option (B) is correct.

Q.20. Consider the following optimization problem:

Maximize z = 4x + 2y

subject to

x + 2y + S1 = 5,

x + y + S2 = 4,

x, y, S1, S2  0.

Initial simplex table is obtained as

| Basic | x | y | S1 | S2 | Solution |
| --- | --- | --- | --- | --- | --- |
| z | 4 | 2 | 0 | 0 | 0 |
| S1 | 1 | 2 | 1 | 0 | 5 |
| S2 | 1 | 1 | 0 | 1 | 4 |

In the first iteration, the

(A) entering variable is y and leaving variable S2

(B) entering variable is x and leaving variable S2

(C) entering variable is x and leaving variable S1

(D) entering variable is y and leaving variable S1

Sol. For a maximization problem, the entering variable is the variable having the most negative coefficient in the z-row. In the table, the most negative coefficient is −4. Hence, entering variable is x. Applying the minimum ratio test, S1: (5/1) = 5, S2 = (4/1) = 4. By minimum ratio rule, S2 is leaving. Hence, option (B) is correct.